Box or Package Vending / Caterpillar / Belt Drive Motor

Multipurpose DC Gear Motor for Vending Machines

| Model No. | Specification |  |  |
| --- | --- | --- | --- |
| Part Number | 301604-003 | Max Eff. Speed | 22 RPM ± 10% |
| Voltage Range | 9.00 – 15.00 VDC | Max Eff. Current | 0.570 Amp |
| Operating Voltage | 12.00 VDC | Max Eff. Torque | 11.25 Kg-cm |
| Reduction | 1:322 | Max Eff. Power | 2.60 W |
| Direction of Rotation |  | Max Power | 4.80 W |
| No Load Speed | 27 RPM ± 10% | Max Power Speed | 14 RPM ± 10% |
| No Load Current | 0.150 Amp | Max Power Current | 1.380 Amp |
| No Load Power | 1.80 W | Max Power Torque | 33.76 Kg-cm |

CONNECTION SYSTEM

Pin 1 – Motor 12 VDC +

Pin 2 – Switch NC Empty

Pin 3 – Switch NO Empty

Pin 4 – GND

Pin 5 – GND

Pin 6 – Motor ON (GND)

| 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- |