Bill To Bill ATM Manual

Contents

I: Preface···························································P2

II：Summery························································P2

III：Safety Notice····················································P2

IV：Machine Picture··················································P3

V：Machine Body and Accessory·······································P3

VI：Configuration····················································P4

VII: Features························································P4

VIII：Functions······················································P5

IX: Keyboard’s Keys and Definitions·····································P5

X: How to Enter into Menu············································P 6

1.Account Check····················································P 6

2.Account Clear·····················································P 6

3. Set Bill Value·····················································P 7

- How to set to accept 1-100 USD····································P 7

- How to set to accept 10-100 USD···································P 8

4. Rate Setting·······················································P 8

- Rate 1:1 Setting·················································P 8

- Rate 1:2 Setting·················································P 9

- Irregular Rate Setting············································P 11

5. Set Password·····················································P 12

6. Recover System··················································P 12

7. Dispense All the bills·············································P13

8. Exit Menu·······················································P 13

9. Check ID························································P13

10. Language·······················································P13

11. Bill Manage····················································P14

12.Set Manage PW··················································P14

13. Set Configuration Password········································P15

XIII：Troubleshooting···············································P15

I：Preface

Dear Customers：

Thanks for your buying,in order to offer an excellent service to you, please read this manual carefully prior to using and please keep it welly.We offer two years period warranty and the life-long maintenance to the products you buy.We will charge the material costs according to the following situations:

- Out of Warranty Period;

- Damages caused by buyers during transportation and loading.

- Errors caused by dis-assembly repair and/or modification without permissions from factory.

II：Summary

The bill Changer model LCD is a machine like an ATM offering a self service of buying the bills/tokens with banknotes. It is widely used in banking financial system, public bus transportation, metro, amusement and laundry shop etc. The machine is working stably and perfectly without labors. LED + LCD displays offer you a direct looking and easy operation.It is your good choice when you want to buy a bill changer to replace human labors to decrease the cost. In additions, you could publish the advertisement on the LCD too to attract audience.

III：Safety Notice

It is strongly recommended to use the leakage protector and the 3-pin plug for the machine so as to avoid static shock .Make sure that there is no other substances like different size bills, screws, cigarette etc so as to avoid incorrect counting, bill jam and damage to the machine.The machine should be placed levelly during operating. It is recommended to put it in a dry, cool and wind-flowing area. It is not allowed to start or stop the machine by plugging or un-plugging the power cable directly, instead you should use the switch on the machine. Keep children away from the machine and do not allow the untrained personnel to open and disassemble the machine.

IV: Machine Picture

V：Machine Body and Accessory

| S/N | Name | QTY | Remarks | Picture |
| --- | --- | --- | --- | --- |
| 1 | Bill To Bill Exchange Machine | 1 PCS | With 1 Bill Acceptor and 1 note dispenser |  |
| 2 | Power Cable | 1 PCS | 3 legs Plug, U.S., Britain, EU etc for your choice |  |
| 3 | Manual | 1 PCS | Chinese or English |  |
| 4 | Key | 3 PCS | 1-Door，1-Cash box，1-bill Container |  |

VI：Configuration

| S/N | Name | QTY | Remarks |
| --- | --- | --- | --- |
| 1 | Motherboard | 1 | Controls the machine’s operation |
| 2 | Bill Acceptor | 1 | For accepting and validating bills, support more than 150 countries currencies. |
| 3 | Note Dispenser | 1 | Dispense Bills |
| 4 | LCD | 1 | Display Menu for A/C checking, Password Setting etc |
| 5 | LED | 1 | To display the inserted bill amount and the dispensed bills |
| 6 | Mini Keyboard | 1 | Used to enter into LCD Menu and parameter setting |
| 7 | Power Cable | 1 | 3-leg-plug, available with U.S. Britain, EU etc110V~240V |
| 8 | Fuse | 1 | 2A Fuse to protect the machine perfectly |
| 9 | Filter | 1 | To filtrate the input110V~240V, avoid the clutter’s disturbance |
| 10 | Switch | 1 | Bring convenience to power turn on and off |

VII：Features

| S/N | Name | Remarks |
| --- | --- | --- |
| 1 | Denominations Supported | 1、5、10、20、50、100 USD/USD/ER··· |
| 2 | Bill Box Capacity | About 500 Bills |
| 3 | Note Dispenser Capacity | About 1000 Bills |
| 4 | Note Size | Width 66~72MM Length: 120~160MM Thickness: 0.08~0.125 |
| 5 | Dispensing Speed | 2.5 Bills/Sec |
| 6 | Volt | 110V~240V |
| 7 | Power Consumption | 100W |
| 8 | Machine Size | 44（L）* 37（W）* 65（H）CM |
| 9 | N.W | 40Kg |

VIII：Functions

- To offer the self service of exchanging bills to bills. e.g. $100 = $20 * 5.

2. Supports more than 150 countries currencies.

3. 13 functions to meet up all necessary requirements, allowing operators to set any results according to their needs.

4. The eye-catching LED shows the inserted bill amount and the due dispensed bills, easy to understand.

5. The big LCD shows you the menu and the parameters, with the mini keyboard, you could easily operate and set all the parameters.

IX: Keyboard’s Keys and Definitions

Keyboard and Keys：Each bill Changer is enclosed with a small keyboard for Menu Operation. There are 18 keys on the keyboard as follows: the numeric keys from【0】to【9】, and other keys【-】,【+】,【Enter】、【Del】、【Back Space】、【Num Lock】、【/】、【*】

b.  Keys Not Used：【Num Lock】、【/】

c.  Definition of the Keys

【0】to【9】：For selecting and inputting numbers.

【-】： scroll up.

【+】：scroll down.

【Enter】：Confirm and enter into menu

【Del】：Cancel and exit menu

【Back Space】: backward/exit

【*】：means 10, like if you want to input 11, press key 【*】and then 【1】

X：  How to Enter into Menu:

The LCD will display 『Welcome』when power is on. Press the key【Enter】, system will display『Please Input Password』, you could enter into the menu after inputting the correct password.

1.Account Check：You could check the record of current account as well as the TB (Total Bills) and TC (Total bills). Look at the following example. (Rate: 1:2 )

|  | Number Displayed | Meaning |
| --- | --- | --- |
| 1. Bill In | 100 | The bills amount of current account |
| 2. bill Out | 200 | The dispensed bills of current account |
| 3. TB | 5800 | The total amount of all the inserted bills up to now. |
| 4. TC | 11600 | The total quantity of all the payout bills up to now. |

2.Account Clear：To clear the records of Bill In and the bill Out to zero.

| Clear Account ? | The system will require your final confirmation. |
| --- | --- |
| 1. Yes | Press key 【1】 to clear the record to 0. Go back to 【Account Check】and you will find that 【Bill In】and【bill Out】will show the number ''0'', that means you have succeeded in clearing the record of the number ''100'' and '' 200 '' |
| 2. No | Press 【2】 to cancel your choice. The system will return back to the previous page. Go back to 【Account Check】and you will find that 【Bill In】and【bill Out】will still show the number ''100'' and ''200'', that means you cancel the choice of '' Clear '' and the record will still remain. |
| Note: TB and TC cannot be cleared to zero ‘0’. |  |

3.Set Bill Value：This function allow you to set to accept any denominations of the banknotes you wish. (e.g.: take USD as example to show how to do the setting) Default Password: 『111111』

| How to set to accept 1, 5, 10, 20, 50, 100 USD |  |  |
| --- | --- | --- |
| Bill Type | Bill Value | Operation Instruction |
| 1.Bill1 | 1 | Press key 【1】 and then input the number 1 |
| 2.Bill2 | 5 | Press key 【2】 and then input the number 5 |
| 3.Bill3 | 10 | Press key 【3】 and then input the number 10 |
| 4.Bill4 | 20 | Press key 【4】 and then input the number 20 |
| 5.Bill5 | 50 | Press key 【5】 and then input the number 50 |
| 6.Bill6 | 100 | Press key 【6】 and then input the number 100 |
| 7.Bill7 |  |  |
| 8.Bill8 |  |  |
| 9.Bill9 |  |  |

| How to set to accept 10, 20, 50, 100 USD. (Reject $ 1 and 5) |  |  |
| --- | --- | --- |
| Bill Type | Bill Value | Operation Instruction |
| 1.Bill1 | 0 | Press key 【1】 and then input the number 0, when you insert $ 1, the bill acceptor will reject it back to you. |
| 2.Bill2 | 0 | Press key【 2】 and then input the number 0,when you insert $ 5, the bill acceptor will reject it back to you. |
| 3.Bill3 | 10 | Press key 【3】 and then input the number 10 |
| 4.Bill4 | 20 | Press key 【4】 and then input the number 20 |
| 5.Bill5 | 50 | Press key 【5】 and then input the number 50 |
| 6.Bill6 | 100 | Press key 【6】 and then input the number 100 |
| 7.Bill7 |  |  |
| 8.Bill8 |  |  |
| 9.Bill9 |  |  |

It allows you to set to accept USD 10 to 100, or 20-100, or 50 - 100 or just 100 only as you wish.

4.Rate Setting

This B2B ATM allows you to realize any rate of exchange. For example you could set it to 1:1 that means when insert USD 1, the machine will dispense 1 $ to customer. Of course you could also set the Rate Setting as 1:2, 1:3, 1:4, 1:5 ···or Irregular Rate. It is easy to understand and operate after reading the following examples. Default Password: 『111111』

| 1:1 Rate Setting |  |  |  |
| --- | --- | --- | --- |
| Bill Type | Payout Bills | Operation Instruction | Meaning |
| 1.Bill1 | 1 | Press key 【1】and then input the number 1 | Inserting $ 1 and you will get 1 bill. |
| 2.Bill2 | 5 | Press key 【2】and then Bills input the number 5 | Inserting $ 5 and you will get 5 bill. |
| 3.Bill3 | 10 | Press key 【3】and then input the number 10 | Inserting $ 10 and you will get 10 bill. |
| 4.Bill4 | 20 | Press key 【4】and then input the number 20 | Inserting $ 20 and you will get 20 bill. |
| 5.Bill5 | 50 | Press key 【5】and then input the number 50 | Inserting $ 50 and you will get 50 bill. |
| 6.Bill6 | 100 | Press key 【6】and then input the number 100 | Inserting $ 100 and you will get 100 bill. |
| 7.Bill7 |  |  |  |
| 8.Bill8 |  |  |  |
| 9.Bill9 |  |  |  |

| 1:2 Rate Setting |  |  |  |
| --- | --- | --- | --- |
| Bill Type | Payout Bills | Operation Instruction | Meaning |
| 1.Bill1 | 1 | Press key 【1】and then input the number 2 | Inserting $ 1 and you will get 2 bill |
| 2.Bill2 | 5 | Press key 【2】and then input the number 10 | Inserting $ 5 and you will get 10 bills. |
| 3.Bill3 | 10 | Press key 【3】and then input the number 20 | Inserting $ 10 and you will get 20 bills. |
| 4.Bill4 | 20 | Press key 【4】and then input the number 40 | Inserting $ 20 and you will get 40 bills. |
| 5.Bill5 | 50 | Press key 【5】and then input the number 100 | Inserting $50 and you will get 100 bills. |
| 6.Bill6 | 100 | Press key 【6】and then input the number 200 | Inserting $100 and you will get 200 bills. |
| 7.Bill7 |  |  |  |
| 8.Bill8 |  |  |  |
| 9.Bill9 |  |  |  |

By analogy the way of setting the rate of 1:3, 1:4, 1:5 ·····is the same.

| Irregular Rate Setting: It could meet the different needs of different locations according to its different situations. It is flexible and suitable for Promotion Activity. For example if you want $100 = $20 *5. Check the following setting example.Of course you could set any number as you wish. See the following example. |  |  |  |
| --- | --- | --- | --- |
| Bill Type | Payout bills | Operation Instruction | Meaning |
| 1.Bill1 | 0 | Press key 【1】and then input the number “0” | Inserting USD 1 and you will get 0 bill |
| 2.Bill2 | 0 | Press key 【2】and then input the number “0” | Inserting USD 5 and you will get 0 bills. |
| 3.Bill3 | 0 | Press key 【3】and then input the number “0” | Inserting USD 10 and you will get 0 bills, 0 more bills to attract customers. |
| 4.Bill4 | 0 | Press key 【4】and then input the number “0” | Inserting USD 20 and you will get 0 bills,0 more bills to attract customers. |
| 5.Bill5 | 0 | Press key 【5】and then input the number “0” | Inserting USD 50 and you will get 0 bills, 0 more bills to attract customers. |
| 6.Bill6 | 5 | Press key 【6】and then input the number “5” | Inserting USD 100 and you will get 5 bills, 0 more bills to attract customers. |
| 7.Bill7 |  |  |  |
| 8.Bill8 |  |  |  |
| 9.Bill9 |  |  |  |

5. Password Setting: Press key 【5】and the LCD will require you as: 『Please Input Old Password』(the default password is 123456)

******

『Please Input New Password』(For example 654321)

******

『Please Input New Password Again』(654321)

******

Then the LCD will display 『New Password OK』, and you have to input new password when you want to enter into Menu next time. If you do not input correctly, it will show 『Wrong Password』

6.Recover System: This function allows you to return the system back to normal working conditions caused by some abnormalities such as:

- Not Enough bills for the Corresponding Inputted Bills

- bill Jam

- Power Off Suddenly During Dispensing.

Operation Instructed:  Press key 【6】 to enter into the sub-menu as follows:

- bill Un-dispensed:  *** (Note: it will display the quantity of bills not dispensed out due to some errors)

- Dispense bills ?

1. Yes -----Press key 【1】and the machine will dispense the balance bills out.

2. No------Press key 【2】and it will not dispense the balance bills but return back to the previous page.

(Note: by both choices, the machine will recover the error and return back to normal working condition)

7. Dispense All the bills：Press key 【7】and the machine will dispense all the bills out of  the container completely. This function is usually used when you want to know how many bills were left in the container.

8. Exit Menu : Press key 【8】and it will return back to 『 Welcome 』

9. Machine ID: Press key【9】to enter into Machine ID and you could see it as follows:

ID: WINFO1405010200000001  （Note: This is the machine ID, each machine is with a unique ID.）

TIME: 20140501  (Note: This is the Production Time, meaning 2014/05/01)

10. Language: (Operation Instruction: Press key【*】and then press key【0】to enter into the following page.Note: key 【*】indicates 10)

1、中文 2、English

11. bill Manage：(Operation Instruction: Press key【*】and then press key【1】to enter into this setting page. Note: key 【*】indicates 10. Default Password: 『111111』)

- Remain：9950 （means it has dispensed 50 bills out. 10000 - 50 = 9950）

- Add Num.：10000（means you add 10000 bills into the container. You could input any number, it = bills you put into the container）

- Alarm Num.：200 （When the remain bills go down from 10000pcs to 200pcs. The machine will send SMS to your mobile or call your mobile to remind you of Insufficient bills. It could continue to work until it reaches Stop Num. 100 pieces which it will stop accepting bills.The same, you could set any alarm number, it all depends on your business situation.）

- Stop Num.：100 （When the remain bills go down to 100pcs, the machine will disable bill acceptor and it won’t accept any bills until you come to recover it and set it to normal working conditions. This purpose is to avoid the COMPLAINT and TROUBLE caused by such case as: if customer inserts $100, and the machine have less than 100pcs bills, in this case, customer would complain or probably strike the machine.The same, you could set any stop number, but it is recommended to set the number equal or higher than the biggest denomination.Take $100 for example, rate is 1:1, then the stop number≧100）

12.  Set Manage PW：

This Password is used to enter into the menu of bill Manage. Press key 【*】and 【2】to enter into the sub menu as follows:

『Please Input Old Password』(the default password is 111111)

******

『Please Input New Password』(For example 654321)

******

『Please Input New Password Again』(654321)

******

Then the LCD will display 『New Password OK』, and you have to input new password when you want to enter into bill Management next time. If you do not input it correctly, it will show 『Wrong Password』

13. Set Config PW

This Password is used to enter into the menu of 3.Set Bill Value and 4.Rate Setting. Press key 【*】and 【5】to enter into the sub menu as follows:

『Please Input Old Config Password』(the default password is 111111)

******

『Please Input New Config Password』(For example 654321)

******

『Please Input New Config Password Again』(654321)

******

Then the LCD will display 『New Config Password OK』, and you have to input new password when you want to enter into bill Management next time. If you do not input correctly, it will show 『Wrong Password』

XIII: Troubleshooting

| Error Described | Causes | Solutions | Remarks |
| --- | --- | --- | --- |
| Failed to communicate with BA | 1：Communication Cable is slack. 2：BA sensor is dirty with dusts. | 1：Re-connect communication cable firmly. 2：Clean the sensor with dry cloth. | Contact service center if the errors are still after following the solutions. |
| Hopper Error | 1：No bill in hopper. 2：bill Jam in hopper. | 1:Refill the bills 2：Remove the jammed bill will be OK. | As above |
| BA Sensor Error | 1：BA sensor is dirty with dust。 2：BA sensor is broken。 | 1：Clean the sensor with dry cloth. 2：Contact with service center to solve it。 | As above |
| BA Error | 1：BA sensor is dirty with dust。 2：BA sensor is broken。 | 1：Clean the sensor with dry cloth. 2：Contact with service center to solve it。 | As above |
| Hopper Sensor Error | 1：Hopper cable is slack or disconnected. 2：Hopper sensor is dirty with dusts. | 1：Re-connect communication cable firmly and then press key ENTER to restore the system. 2：Clean the sensor with dry cloth. | As above |
| Some bills were not dispensed due to some errors, please enter into Menu | The machine was suddenly powered off during dispensing the bills. | Press key 【Enter】, input password to enter into menu. Select the No.6--Recovery to enter into a sub-menu, select ‘YES’ or ‘NO’ to restore the system. | As above. |

Please give the machine at least 30 seconds of rest when you turn on the power again.